Unit A: Lesson 3.2  /  70mins  /  15-24yrs

Dimensions, Scales, and Drawing Symbols

Learning Objectives

Interpret dimensions, scales, and drawing symbols in technical drawings Identify annotations such as hole specifications, tolerances, and surface finishes Recognise how material types, scale, and title block information inform manufacturing decisions

Success Criteria

By the end of this lesson, students will be able to: - Explain key dimensioning conventions (e.g. radius, diameter, hole depth) - Identify title block elements and interpret scale, author, and material info - Recognise drawing symbols and annotations (e.g. surface finish, tolerances) - Analyse technical drawings to detect potential hazards and suggest control measures

| Core Resources | Additional Resources |
| --- | --- |
| PowerPoint: 3.2 Dimensions, Scales, and Symbols Technical Drawing – Punch Body | Video: How to Read an Engineering Drawing |

| Preparation Checklist – Before the Lesson |  |
| --- | --- |
|  | Prepare drill gauge or physical object for warm-up |
|  | Load video and test audio playback (optional) |
|  | Print copies of the Punch Body drawing for annotation |
|  | Review key dimensioning terms and standards to explain during modelling |

| Description | Resource |
| --- | --- |
| 1.1 #### Warm-up / Duration: 15mins ##### Purpose: To spark curiosity and highlight the importance of dimensions in interpreting technical drawings. ##### Pedagogical Approach: Individual + class discussion ##### Activity - Show students the activity slide, asking if they could make this accurately and what they think is missing. - Have students guess the dimensions and sketch what they think the object looks like at full size - Invite volunteers to share their guesses | 3.2 Slides |
| 1.2 #### Dimensions, Scales and Title Blocks / Duration: 25mins ##### Purpose: To introduce students to the conventions and importance of dimensions, scales, and title blocks in engineering drawings. ##### Pedagogical Approach: Direct instruction with visual examples ##### Dimensioning - Show slides introducing dimensioning basics: linear, diameter, radius, hole depth, angles. - Explain dimensioning symbols and standards (e.g., mm unit convention, leader lines, center marks). - Optional: Show video (focusing on title block, scale, types of lines, dimension chapters). Add a brief guided question during the video or pause to clarify key points. ##### Title Block and Scale - Explain the title block: project name, scale, material, tolerances, author, date. - Show an example drawing (paper punch) and walk through the information provided in both the drawing and title block. - Have students annotate a printout or on-screen version of the drawing by identifying each part of the title block. | 3.2 Slides Technical Drawing – Punch Body Video |
| 1.3 #### Hazard Identification from Drawings / Duration 30mins ##### Purpose: To build students’ ability to interpret technical drawings for potential hazards and link them to workplace safety practices. ##### Pedagogical Approach: Guided instruction with visual and applied practice ##### Activity - Explain how technical drawings can reveal potential hazards, such as sharp edges, pinch points, or incorrect material selection. - Discuss how dimensions and features (e.g., drilled holes, surface finish, chamfers) may impact safety. - Show the Punch Body drawing and ask: - What materials are involved? - What processes are likely used? - What hazards might these create (e.g. sharp edges, swarf, heat)? - Think–Pair–Share: Students reflect on a drawing and identify two possible hazards and related control measures. - Class discussion to compile a shared list of hazards and safety controls (e.g. PPE, guarding, SOPs). | 3.2 Slides Technical Drawing – Punch Body |

Teacher Notes and Reflection

What worked well? What would you do differently next time? Any ideas for extending, enriching, or scaffolding learning?