Unit A: Lesson 4.4  /  70mins  /  15-24yrs

Marking Out the Drill Gauge

Learning Objectives

- By the end of this lesson, students will: - 1. Interpret technical drawings to identify hole locations and cut lines. - 2. Use marking out tools (rule, square, divider, vernier height gauge) accurately. - 3. Centre punch all hole locations with correct alignment and spacing. - 4. Prepare workpieces for drilling based on the marked layout. - Success Criteria - I will successfully: - • Transfer all hole and profile measurements accurately from drawing to material. • Use the correct tools and techniques for layout work. • Centre punch clearly and accurately at all hole locations. - Related Assessment Task(s) Assessment 1: Practical Demonstration

| Core Resources | Additional Resources |
| --- | --- |
| - Technical drawing (Drill Gauge) | - Sample layout on whiteboard or printed example |

| WHS Considerations | Tech Alternatives |
| --- | --- |
| - Eye protection when centre punching - Careful use of sharp scribes and dividers | - Annotated drawing handouts if digital display unavailable |

| Required Tools, Equipment, Materials & PPE |  |
| --- | --- |
| Tools | Steel rule, scribe/scratch gauge, centre punch, square, divider, vernier height gauge |
| Equipment | Marking surface, bench |
| Materials | Marking blue or permanent marker |
| PPE | Safety glasses, steel-capped boots |

Optional Homework

| Description | Resource |
| --- | --- |
| 1.1 #### Introduction and Drawing Interpretation: 10mins ##### Activity Instructions Recap: Why accuracy in marking matters Discuss hole size, spacing, and cut line requirements Students review specification sheet with teacher guidance |  |
| 1.2 #### I Do – Demonstration – Duration: 15mins ##### Activity Instructions Demonstrate applying marking blue or permanent marker Show how to use rule and square to mark base lines Use divider and vernier height gauge to locate hole centres Demonstrate safe, accurate centre punching at intersections Emphasise tool grip, control, and double-checking layout | - Marking blue or permanent marker - Rule and square - Divider and vernier height gauge |
| 1.3 #### We Do – Guided Practice: 15mins ##### Activity Instructions: Students mark out 1 hole location as a class step-by-step Teacher circulates to check technique, give tips Peer discussion: “Is this aligned?” “How would you check again?” |  |
| 1.4 #### You Do – Independent Marking: 25mins ##### Activity: Students mark all holes and profile lines on their workpiece Centre punch all hole centres once verified Teacher checks layout and signs off before next practical step |  |
| 1.5 #### General Housekeeping & Reflection: 5mins ##### Activity Instructions Return all layout tools Wipe work areas clean Exit prompt: “What did you check twice before punching?” |  |