Vertical Mill SOP & Demonstration

Unit A Worksheet 5.11

| Student Name | Date | Class |
| --- | --- | --- |
| Sienna M. | 19/9/25 | Yr 11 Engineering Skills |

#### Instructions

You will observe a live demonstration of vertical milling procedures, including machine setup, safe startup, correct handling, and shutdown. You will also begin applying the SOP in hands-on milling.

Use this worksheet to take notes during the demo and check your understanding before operating the machine.

#### Section 1 – Milling SOP Overview

Confirm the workpiece is clamped securely in the vice and the spindle path is clear.

Always wear PPE including safety glasses, apron, and enclosed shoes before approaching the machine.

Write down three key safety checks or actions shown in the demo:

Stand to the side during spindle startup and know the location of the emergency stop.

1.______________________________________________________________________________________

2.______________________________________________________________________________________

3.______________________________________________________________________________________

Check that the tool is installed correctly and tightened using the drawbar, confirm spindle direction matches the cutter type (clockwise for standard end mills), set appropriate spindle speed and feed rate based on material, clamp the workpiece securely and zero the DRO, stand clear and start the spindle, then engage feed manually or automatically.stop.

Section 2 – Startup & Shutdown Sequence

List the correct startup steps based on the demonstration:

Retract the tool from the workpiece and stop the spindle, turn off the power and clean the machine bed, remove chips using a brush (not hands) and wipe down surfaces, return tools and cutters to storage and report any issues to the teacher.

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

List the correct shutdown steps after milling is complete:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

#### Section 3 – Milling Safety Checklist

Before operating the mill, confirm each item:

PPE worn (glasses, apron, enclosed shoes)

Workpiece securely clamped

Spindle and tool path clear

No loose clothing or gloves

Emergency stop location known

Teacher briefed before milling?    Yes   ☐ No

#### Section 4 – Reflection & Readiness

I feel confident identifying the X, Y, and Z axes and setting up the vice and workpiece correctly. I also understand how to check spindle direction and use the DRO to control depth and position.

What part will you be milling first?  ☐ Head    Handle end     ☐ Other:_________________________

What are you most confident about after the demo?

______________________________________________________________________________________________________________________________________________________________________________

I want to double-check the spindle speed settings for aluminium and confirm the tool is tightened properly. I’ll also ask my teacher to verify my clamping setup before I begin milling.

What do you want to double-check before operating the machine?

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________